Haber-Bosch process:

(1) H2 production:  CH4 + H2O → CO + 3H2 (after removing S impurities from methane (CH4)), and

CO + H2O → CO2 + H2

(2) Ammonia production: 3H2 + N2 → 2NH3 - Using iron catalyst at 400 – 500°C and pressures above 100 bar with recycling of the unconverted part of the synthesis gas and separating the ammonia product under high pressure. Since the 1990s a ruthenium (Ru)-based catalyst was introduced which allows slightly lower synthesis pressure and is used in a few world-size plants.

(3) production of fertilizers (e.g., ammonium nitrate, urea)

(1) https://en.wikipedia.org/wiki/Haber_process; (2) https://onlinelibrary.wiley.com/doi/full/10.1002/14356007.o02_o11

“Ruthenium is one of the rarest metals on Earth. It is obtained commercially from the wastes of nickel refining.” - https://www.rsc.org/periodic-table/element/44/ruthenium#:~:text=is%20highly%20toxic.-,Natural%20abundance,the%20wastes%20of%20nickel%20refining.

https://onlinelibrary.wiley.com/doi/full/10.1002/14356007.o02_o11